# Saab 91B Safir	PAGE 6

EMERGENCY AND ABNORMAL CHECKLIST

ENGINE FIRE ON START-UP

	1	THROTTLE	FULLY OPEN	1
	2	STARTER	CONTINUE TO CRANK	2

		IF UNSUCCESSFUL		

	3	FUEL COCK	CLOSED	3
	4	MIXTURE	CUT-OFF	4
	5	FUEL PUMP	OFF	5
	6	AIRCRAFT	EVACUATE	6
	7	FIRE EXTINGUISHER	ACTIVATE	7

ENGINE FIRE IN FLIGHT

	1	MIXTURE	CUT-OFF	1
	2	THROTTLE	IDLE	2
	3	FUEL COCK	CLOSED	3
	4	FUEL PUMP	OFF	4
	5	CABIN HEATER AND VENTS	OFF	5
	6	PERFORM POWER-OFF LANDING		6

ELECTRICAL FIRE 

	1	BATTERY MASTER AND ALTERNATOR	OFF	1
	2	ALL ELECTRICAL EQUIPMENT	OFF	2
	3	CABIN VENTS	OPEN	3
	4	CABIN HEATER	OFF	4
	5	LAND ASAP		5

POWER LOSS DURING DEPARTURE

	1	SUFFICIENT RUNWAY AHEAD	GEAR DOWN, LAND	1
	2	ROUGH TERRAIN OR OBSTACLES	GEAR UP, LAND STRAIGHT	2

		SUFFICIENT ALTITUDE HAS BEEN REACHED FOR A RESTART	

	3	AIRSPEED	81 KIAS	3
	4	MIXTURE	RICH	4
	5	FUEL PUMP	ON, PRESS CHCKD	5
	6	ALTERNATE AIR	ON	6
	7	MAGNETOS	CHCKD	7

		IF POWER IS NOT RESTORED		

	8	PREPARE A POWER-OFF LANDING		8

Saab 91B Safir	PAGE 7

POWER LOSS IN FLIGHT

	1	AIRSPEED	81 KIAS	1
	2	LANDING AREA	FIND	2
	3	MIXTURE	RICH	3
	4	FUEL PUMP	ON, PRESS CHCKD	4
	5	ALTERNATE AIR	ON	5
	6	MAGNETOS	CHCKD	6
	7	ENGINE INSTRUMENTS	CHCKD	7

		IF POWER IS NOT RESTORED		

	8	PERFORM POWER-OFF LANDING		8

POWER-OFF LANDING

	1	AISPEED	81 KIAS	1
	2	LANDING AREA	FIND	2
	3	EMERGENCY CALL	TRANSMIT	3
	4	GEAR	AS REQUIRED	4
	5	FLAPS	AS REQUIRED	5
	6	AIRSPEED GEAR & / OR FLAPS DOWN	73 KIAS	6
	7	AIRSPEED GEAR & FLAPS UP	81 KIAS	7

		WHEN COMMITTED TO LAND		

	8	MIXTURE	CUT-OFF	8
	9	THROTTLE	IDLE	9
	10	AVIONICS & ELECTRICAL SWITCHES	OFF	10
	10	BATTERY MASTER AND ALTERNATOR	OFF	11
	12	MAGNETOS	OFF	12
	13	FUEL PUMP	OFF	13
	14	FUEL COCK	CLOSED	14
	15	SEAT BELT AND HARNESS	TIGHT	15

LOSS OF OIL PRESSURE

	1	OIL TEMPERATURE	MONITOR	1
	2	LAND ASAP		2
	3	PREPARE FOR A POWER-OFF LANDING		3

HIGH OIL TEMPERATURE

	1	MIXTURE	RICH	1
	2	THROTTLE	REDUCE	2
	3	COWL FLAPS	OPEN	3
	4	OIL PRESSURE	MONITOR	4
	5	OIL TEMPERATURE	MONITOR	5
	6	LAND AS SOON AS PRACTICAL		6

Saab 91B Safir	PAGE 8

LOSS OF FUEL PRESSURE

	1	FUEL PUMP	ON, PRESS CHCKD	1
	2	FUEL SELECTOR	SWITCH TANK	2
	3	FLOW REGAINED	LAND ASAP	3
	4	FLOW NOT REGAINED	PREPARE PWR-OFF LANDING	4

PROPELLER OVERSPEED

	1	THROTTLE	RETARD	1
	2	AIRSPEED	RETARD	2
	3	OIL PRESSURE	CHECK	3
	4	PROP	FULL LOW RPM	4
	5	SET PROP TO POSITION GIVING CONTROL		5
	6	THROTTLE AS REQ TO REMAIN BELOW 2700 RPM	6
	7	LAND ASAP		7

ELECTRICAL FAILURE

	1	AMMETER	VERIFY ALT FAILURE	1
	2	ELECTRICAL LOAD	REDUCE	2
	3 	ALTERNATOR FIELD CB	CHECK IN	3
	4	ALTERNATOR OUTPUT SWITCH	CHECK ON	4
	5	ALTERNATOR MASTER SWITCH	OFF FOR 1 SEC, THEN ON	5
	6	AMMETER	CHECK ALT OUTPUT	6

		IF NO OUTPUT INDICATED		

	7	ALTERNATOR MASTER SWITCH	OFF	7
	8	ELECTRICAL LOADS	REDUCE TO A MINIMUM	8
	9	VOLTMETER	OBSERVE	9
	10	LAND AS SOON AS PRACTICAL		10

ELECTRICAL OVERLOAD (10 AMPS above normal)

	1	BATTERY MASTER	OFF	1

		IF AMMETER READING DOES NOT DECREASE		

	2	ALTERNATOR MASTER SWITCH	OFF	2
	3	LAND AS SOON AS PRACTICAL		3

		IF AMMETER READING DOES DECREASE		

	2	BATTERY MASTER	ON	2
	3	AMMETER	MONITOR	3
	4	LAND AS SOON AS PRACTICAL		4