# Saab 91B Safir	PAGE 1

NORMAL OPERATION CHECKLIST

PREFLIGHT CHECK

	1	FLIGHT NOTIFICATION	FILED	1
	2	AIRCRAFT PAPERS	CHCKD	2
	3	AIRCRAFT LOG & FLIGHT TIME INDICATOR	CHCKD, NOTED	3
	4	LOADSHEET	CHCKD	4
	5	OUTSIDE CHECK	COMPLETED	5
	6	TOW BAR	SECURED	6
	7	CABIN	CHCKD	7
	8	CONTROLS	FREE & CORRECT	8

CHECK BEFORE ENGINE START

	1	CANOPY	CLOSED	1
	2	PARKING BRAKE	SET	2
	3	SEATS	ADJUSTED	3
	4	SEAT BELT AND HARNESS	FASTENED	4
	5	ELECTRICAL EQUIPMENT	ALL OFF	5
	6	BAR SWITCHES	CHCKD ON	6
	7	CIRCUIT BREAKERS	ALL IN	7
	8	AVIONICS (5)	OFF	8
	9	GEAR	DOWN	9
	10	BATTERY MASTER AND ALTERNATOR	ON	10
	11	GEAR LIGHTS	CHCKD	11
	12	FUEL QUANTITY	RECORD, L+R, ENDURANCE	12
	13	ALTERNATE AIR	OFF	13
	14	THROTTLE	IDLE	14
	15	PROP	HIGH RPM	15
	16	MIXTURE	CUT-OFF	16
	17	COWL FLAPS	OPEN	17

STARTING ENGINE WHEN COLD

	1	ANTI–COLLISION LIGHT	ON	1
	2	THROTTLE	OPEN 1 – 2 cm	2
	3	MAGNETOS	M1	3
	4	FUEL PUMP	ON, PRESS CHECKED	4
	5	MIXTURE	RICH, WHEN FF CUT-OFF	5
	6	PROPELLER AREA	CLEAR	6
	7	STARTER	ENGAGE	7
	8	MIXTURE	TO RICH AS ENG FIRES	8
	9	MAGNETOS	M1 + M2	9
	10	THROTTLE	1000 RPM	10
	11	OIL PRESSURE (within 30")	GREEN ARC	11

Saab 91B Safir	PAGE 2

STARTING ENGINE WHEN HOT

	1	ANTI-COLLISION LIGHT	ON	1
	2	THROTTLE	OPEN 1 – 2 cm	2
	3	MAGNETOS	M1	3
	4	MIXTURE	CUT-OFF	4
	5	PROPELLER AREA	CLEAR	5
	6	STARTER	ENGAGE	6
	7	MIXTURE	TO RICH AS ENG FIRES	7
	8	MAGNETOS	M1 + M2	8
	9	THROTTLE	1000 RPM	9
	10	OIL PRESSURE (within 30”)	GREEN ARC	10

CHECK AFTER ENGINE START

	1 	FUEL PUMP	OFF, PRESS CHCKD	1
	2	ALTERNATOR OUTPUT	CHCKD	2
	3	VENTILATION, HEATER	AS REQUIRED	3
	4	POSITION LIGHTS	AS REQUIRED	4
	5	AVIONICS (5)	ON	5
	6	ELT ACTIVITY	CHCKD	6
	7	FLIGHT INSTRUMENTS	CHECK & SET	7
	8	ENGINE INSTRUMENTS	CHCKD	8
	9	AVIONICS NAV, COM	SET & PRESELECTED	9
	10	COWL FLAPS	CLOSED	10

TAXI CHECK

	1	BRAKES	CHCKD	1
	2	GYRO INSTRUMENTS	CHCKD	2

RUN-UP

	1	PARKING BRAKE	SET	1
	2	CYL HEAD AND OIL TEMPERATURE	GREEN ARC	2
	3	ZONE BEHIND AIRCRAFT	FREE	3
	4	THROTTLE	SET 1800 RPM	4
	5	OIL PRESSURE & SUCTION	GREEN ARC	5
	6	MAGNETOS (MAX -175/=50RPM)	CHCKD	6
 	7	PROP CYCLING X3 (-300RPM)	CHCKD	7
	8	PROP STABILIZING (-100RPM + 3InHg)	CHCKD	8
	9	MIXTURE (EGT, FF)	CHCKD	9
	10	ALTERNATE AIR	CHCKD	10
	11	ENGINE INSTRUMENTS	WITHIN LIMITS	11
	12	THROTTLE IDLE	CHCKD (500 - 700 RPM)	12
	13	THROTTLE	SET 1000 RPM	13

Saab 91B Safir	PAGE 3

CHECK BEFORE DEPARTURE PART 1

	1	SEAT POSITION	CHCKD & LOCKED	1
	2	FUEL QUANTITY	CHCKD, ENDURANCE?	2
	3	FUEL SELECTOR	LEFT TANK	3
	4	FRICTION LOCK	SET	4
	5	CONTROLS	FREE & CORRECT	5
	6	TRIMS	SET FOR T/O (2)	6
	7	FLAPS	SET	7
	8	FLIGHT INSTRUMENTS & AVIONICS	SET FOR DEPARTURE	8
	9	CABIN AND PAX	SECURED	9

CHECK BEFORE DEPARTURE PART 2

	1	FUEL PUMP	ON, PRESS CHCKD	1
	2	ALTERNATE AIR	OFF	2
	3	PROP	HIGH RPM	3
	4	MIXTURE	RICH OR AS REQ.	4
	5	MAGNETOS	BOTH	5
	6	ENGINE INSTRUMENTS	CHCKD	6
	7	COWL FLAPS	SET	7
	8	CANOPY AND WINDOWS	CLOSED & LOCKED	8
	9	DEPARTURE BRIEFING	COMPLETED	9

LINE UP CHECK

	1	LANDING LIGHTS	ON	1
	2	TIME	NOTED	2
	3	TRANSPONDER	SBY OR ALT	3
	4	APPROACH SECTOR & RUNWAY	FREE & CHCKD	4

NORMAL TAKEOFF

	1	THROTTLE	ADVANCE TO FULL	1
	2	ROTATE	AT VX	2
	3	SPEED	ACCELERATE TO VY	3
	4	GEAR	RETRACT	4

SHORT FIELD

	1	BRAKES	HOLD	1
	2	THROTTLE	ADVANCE TO FULL	2
	3	BRAKES	RELEASE	3
	4	ROTATE	AT VX	4
	5	SPEED	MAINTAIN VX	5
	6	GEAR	RETRACT	6
	7	FLAPS	RETRACT SLOWLY	7
	8	SPEED WHEN CLEAR OF OBSTACLE	ACCELERATE TO VY	8

Saab 91B Safir	PAGE 4

CLIMB CHECK

	1	GEAR	UP & CHCKD	1
 	2	FLAPS	UP	2
	3	CLIMB POWER (25”/2500RPM)	SET	3
	4	FUEL PUMP	OFF, PRESS CHCKD	4
	5	LANDING LIGHTS	AS REQUIRED	5

CRUISE CHECK

	1	ALTIMETER	SET (STD/QNH)	1
	2	GYRO	CHCKD & SET	2
	3	CRUISE POWER (ACC PWR SETTING TABLE)	SET	3
	4	MIXTURE (LEAN > 5000 ft & PWR < 75 %)	SET	4
	5	FUEL QUANTITY	CHCKD, ENDURANCE	5
	6	LANDING LIGHTS	AS REQUIRED	6
	7	COWL FLAPS	SET	7

DESCENT CHECK

	1	ATIS	NOTED	1
	2	APPROACH BRIEFING	COMPLETED	2
	3	AVIONICS	SET & CHCKD	3
	4	GYRO	CHCKD & SET	4
	5	CABIN AND PAX	SECURED	5

APPROACH CHECK PART 1

	1	ALTIMETER	SET	1
	2	LANDING LIGHTS	ON	2
	3	FUEL PUMP	ON, PRESS CHCKD	3
	4	FUEL QUANTITY	CHCKD, ENDURANCE	4

APPROACH CHECK PART 2

	1	FLAPS	SET	1
	2	GEAR	DOWN & CHCKD	2
	3	COWL FLAPS	SET	3
	4	MIXTURE	RICH / AS REQUIRED	4
	5	ALTERNATE AIR	OFF	5

FINAL CHECK

	1	FLAPS	SET	1
	2	GEAR	DOWN & CHCKD	2
	3 	MIXTURE	RICH / AS REQUIRED	3
	4	PROP	HIGH RPM	4
	5	ALTERNATE AIR	OFF	5
	6	RUNWAY	FREE & CORRECT	6

Saab 91B Safir 	PAGE 5

GO AROUND

	1	THROTTLE	ADVANCE TO FULL	1
	2	FLAPS	START	2
	3	POSITIVE CLIMB	ESTABLISH	3
	4	GEAR	UP	4
	5	FLAPS	UP	5

AFTER LANDING CHECK

	1	TRANSPONDER	SBY	1
	2	TIME	NOTED	2
	3	LANDING LIGHTS	AS REQUIRED	3
	4	FUEL PUMP	OFF, PRESS CHCKD	4
	5	FLAPS	UP	5
	6	COWL FLAPS	OPEN	6

ENGINE SHUT DOWN AND PARKING

	1	PARKING BRAKE	SET	1
	2	THROTTLE	SET 1000 RPM (1 MIN)	2
	3	ELECTICAL ACTIVITY	CHCKD	3
	4	ELECTRICAL CONSUMERS	ALL OFF	4
	5	AVIONICS (5)	OFF	5
	6	MAGNETOS GROUNDING	CHCKD	6
	7	MIXTURE	CUT-OFF	7
	8	MAGNETOS	OFF	8
	9	BATTERY MASTER AND ALTERNATOR	OFF	9
	10	FLIGHT DATAS AND DOCUMENTS	NOTED/COMPLETED	10
	11	PARKING BRAKE	SET AS REQ	11

SPECIAL OPERATION CHECKLIST

STARTING ENGINE WITH EXTERNAL POWER

	1	BATTERY MASTER AND ALTERNATOR	OFF	1
	2	ELECTRICAL EQUIPMENT	ALL OFF	2
	3	EXTERNAL POWER SOURCE	CONNECT AND SWITCH ON	3
	4	START PROCEDURE	COMPLETE	4
	5	THROTTLE	LOWEST POSSIBLE SETTING	5
	6	EXTERNAL POWER SOURCE	SWITCH OFF AND DISCONN.	6
	7	BATTERY MASTER AND ALTERNATOR	ON	7
	8	AMMETER	CHECK	8