# Saab 91B Safir	PAGE 9

OPERATING DATA AND LIMITATIONS

SPEEDS (KIAS)

	VSO	56	VFE	83
	VS1	63	VFO	86
	VX	61	VLE	120
	VY	83	VLO	95
	VA	132	VGLIDE CLEAN	81
	VNO	140	VGLIDE WITH GEAR AND/OR FLAPS	73
	VNE	185	VAPPR	73	Max demonstrated crosswind	17

WEIGHT & BALANCE

	MTOM	1165 kg
	MTOM for aerobatic	1110 kg
	Basic empty mass	720 kg
	C.G. position arm	???? mm
	Max weight for seat	100 kg
	Max weight in luggage hold	20 kg
	Front seats arm	2445 mm
	Fuel arm	2450 mm
	Rear seat arm	3500 mm
	Luggage hold arm	3500 mm

ENGINE

	Maximum horsepower at MSL	190
	Maximum RPM	2700
	Maximum allowable overspeed	2750
	Maximum oil temperature	245 °F
	Desired oil temperature	180 °F
	Maximum cylinder head temperature	500 °F
	Maximum oil pressure	95 PSI
	Minimum oil pressure	55 PSI
	Idling oil pressure	25 PSI
	Maximum allowable overpress	115 PSI
	Starter limitation	30 sec crank / 2 min break
	Maximum oil capacity	8 U.S. Quarts
	Minimum safe oil capacity	2 U.S. Quarts
	Oil grade	SAE15W50

Saab 91B Safir	PAGE 10

FUEL

	Total fuel capacity	175 lit
	Fuel type (minimum octane)	100/130
	Maximum fuel pressure	45 PSI
	Minimum fuel pressure	14 PSI

ELECTRICAL SYSTEM

	Battery voltage	
	Battery amperage	
	Alternator voltage	
	Alternator amperage

GENERAL

	Tyre pressure NLG Min/Max	
	Tyre pressure MLG Min/Max