- Cockpit and Panel for Junkers F 13

- The early Cockpits

The cockpits of the early aircraft did not have much similaritity with the nowadays cockpits. They were almost "cock pits" in the real meaning – a pit where you drop the cocks for their fight. This constructions, very common for the warbirds of WW1, have been used for the first passenger aircraft too.

A real instrument panel was seldom seen, the gauges were mounted directly to the cockpit structure. With these freely mounted gauges, armatures and pipings this cockpits rather remember one on a machine control panel or even on an engineers stand on the steam locomotives. The make and positioning of the gauges in the cockpit were not standardized and often placed randomly. This may look impressive and picturesque today, however influenced the pilots of these days for a quick and safe reading of the gauges. Especially in critical situations additional danger was prone for pilots and passengers.

A nightlighting of the gauges as common today was not available, at that era night flights have not been usually. However the scales and needles were covered with luminous paint, this improved the reading under poor illumination.

- The Cockpit of the Ju F 13

The cockpit of the Ju F13 was such a "cock pit" too, which forced the pilot to enter from top. For the first time however the 2 pilots sitting aside and the forward tilted entrys as well as the roofed cockpit – a new feature – may be interpreted as the predecessor of a window covered cockpit. Comfortable leather chairs have been provided for the pilots. The cockpit openings to the front were cut free with small windshields similar to the aircraft used in WW1.

The gauges and armatures of the Ju F13 were mounted freely to the front structure of the cockpit. This was built up by a framework made from aluminium profiles, the open areas either covered with plain sheet metal or the Junkes typical corrugated aluminium sheet metal.

The Ju F13 did have a single steering column nominally, the rotation and movements were transferred by chain drives and pushrods to the control surfaces. The co-pilots seat was occupied by a mechanic, a navigator or by the fifth passenger usually. On special request a dual steering column was made available.

About the visibility of the pilots from inside the cockpit of the Ju F13 only a few and contradicting reports are known. One pilot reported such a poor sight he had to almost stand upright in difficult approaches to have benefical sight. Another one reported quite good visibilty. One explanation could be, the lack of landing flaps on the Ju F13. This forced, during slow flights and in final approach, a high nose up pitch. The visibility to the front in fact was not available. The second pilot reporting possibly was used to older aircraft, even providing a worse sight to the front , or he was a very tall person ;-) Everything appears relative...

- The Instruments (Gauges) of the Ju F 13

Looking at the gauges, armatures and equipments of the early twenties cockpits a remarkable variety of shapes and makes will be found. Partially the gauges remained in use from before 1920. Standardization was at its beginning, each manufacturer preferred his own, often very lovely, styled design of the gauges. In best case a company standard was used.

Therefore gauges with the same function looked quite different pending on the manufacturer – one time you could find a white scale, another time a black scale, rotation of the pointer needle was ccw, on another it was cw, the zero position was on top , next at bottom of the scale etc.

However, a few of the early "standardization sheets" already existet, here the housings, scales, scale divisions etc. have been defined exactly. For example the oil pressure and fuel status gauge is mentioned here.

The positioning of the gauges in the Ju F13 does not follow any standard. It is however quite readable and does require a short training period only.

There was no night lighting of the gauges as for today – explained above already. Scales and pointer needles at least were coverd by luminous paint (radioactive at that time).

Almost nobody can resist to the charme of this pretty and old gauges. Towards the end of the 20´s the old gauges were replaced by the standardized gauges more and more, as they become typical for cockpits of the 30´s.

- The FS-Cockpit

For the provided cockpit (panel) detailed research about history has been performed. The target was to stay as close as possible to the original for function und visual effects, despite of the compromises needed. The background bitmaps of the panel and the gauge bitmaps are based on historic photographs and conserved original gauges.

The gauges are slightly enlarged versus the original size to improve the readability. The lever for the ignition timing is not represented, since we believe this function will not be supported by the FS. As a substitute the BOSCH magneto switch is placed here – in the original position this switch is much lower and would be placed out of the panel window then.

The steering column is assigned to its own window, you can switch on/off by a mouse click in the area of the handles.

The fuel status gauges are located in the original between the pilots seats on the floor. They are as well in an own window and can be switched on/off by a mouse click in the area of the fuel filter. Here also the fuel tank selector switch is placed in the panel, I assume it was in the lower part of the center console too.

The visibility out of the FS-Cockpit is quite poor compared to todays cockpits, but it is close to what was the reality. Inflight it is acceptable, but for approach and landing as well as for taxiing it is difficult. Improvements are the adjustment of seat position in FS (Shift+Enter) or the external (chase plane) view.

Description of the individual gauges and armatures

.

Magneto Switch

The magneto switch by Bosch company was one of the standard equipmets in aircraft of that era. The housing normally was plain metal surface or sometimes covered by black paint. With the magneto switch the two ignition magneteos of the engine could be switched on and off. The switch positions were 0 - M1 – M2 - Both. The lever was dismountable and formed this way some kind of ignition key. Very often the lever was secured by a bolt and small chain to the switch housing.

To activate the switch perform a mouse klick on the left or right side of the switch. This switch positions are possible:

| 0 | All Magnetose OFF | Engine off |
| --- | --- | --- |
| M1 | Magneto 1 ON | Test Magneto 1 |
| M2 | Magneto 2 ON | Test Magneto 2 |
| 2 | Both Magnetos ON | For engine start and nominal operation |

- Fuel Air Mixture and Throttle

The mixture lever (on top) for the fuel air mixture is built as a push/pull rod with a ball grip.

For adjustment klick upper or lower area of the ball grip with left mouse button. There is no smooth transition by pulling with the mouse button pressed.

The throttle lever (lower position) for the fuel regulation (rpm adjustment) is built up as push/pull rod with a ball grip too.

For adjustment klick upper or lower area of the ball grip with left mouse button. There is no smooth transition by pulling with the mouse button pressed.

- Tachometer

PHYLAX-Tachometer by company Morell. The main divisions of the scale and the pointer needle are covered with luminous paint. The rpm values from 1000 rpm onwards are marked by black fields and the ciphers are in luminous paint too.

The range of the tachometer lasts from 300 rpm to 1600 rpm.

- Starter Magneto

This starter magneto from company Bosch was one of the standard equipments in almost all aircraft of this era. Normally the housing was painted black, some times it was left in metalic surface condition. By rotation of the crank a dynamo was producing the electric power for the engine ignition. Pending on the availability of mounting space inside the cockpit the starter magneto was installed tiltet or even upside down.

To activate the crank press left mouse button in the area of the crank (crank will start to rotate), keep pressed until the engine has started.

- Airspeed Indicator

The airspeed indicator presumably is made by company Bruhn. The reading is in km/h. A red mark indicates the zero or off position of the pointer needle. On the housing rim of the airspeed indicator two marker bugs are available. This bugs are adjustable and usually show the the preferred speed for best climb and best cruise. After adjustment they are fixed in position by a small screw. The marker bugs and the pointer needle are painted in red and have partial areas filled with luminous paint.

The measurement range of the airspeed indicator lasts from 50 to 250 km/h. The first marker bug – value for best cruise – may be adjusted by a mouse click into the left or right UPPER half of the scale. The second marker bug – value for best climb - may be adjusted by a mouse click into the left or right LOWER half of the scale. Quick adjustments are possible with the mouse button kept pressed.

- Altimeter

The altimeter was produced by company Morell and is reading in km. The ciphers, the main divisions of the scale and the small pointer needle tip are covered with luminous paint. For the compensation of the pressure drift an adjustment knop was available, this could be secured by a locking ring. An adjustable marker bug on the outer circumference was usually set by the pilots for the cruise altitude or the altitude of the destination airfield.

The measurement range of this altimeter lasts from 0 to 8 km. For adjustment of the needle press left mouse button on either left or right side of the adjustment knop. To set the marker bug press left mouse button on either left or right side of the scale. Keep mouse button pressed for quick adjustment.

- Compass

This compass can be found in the cockpit of the Junkers Ju A 50, restored in the Deutsches Museum Munich. The ring shaped scale is supported by a cardanic joint to compensate the pitch and roll of the aircraft. In the lower part compensation elements are installed to neutralize the magnetic distortions.

- Clock

The simple and robust clock is a typical aircraft clock as used since the WW1 until early 20´s. The scale and needles are formed pretty nice. The 8-days clock work is a swiss precission make. Usually this fixed mounted clocks were found in larger size aircraft only. In small planes the pilots fixed their own wrist watches or pocket watches in a special clamp and took them out again leaving the aircraft.

This clock here is not built after a specific original, there are details from different clocks. For adjustment of the clock, click left or right side of the knob with the mouse. Keep mouse button pressed for quick adjustment.

- Fuel Tank Selector

For the fuel tank selector switch I could not find any original documents. I have tried to make use of enlarged photographs to pick the details from different cockpits of that era to be able to provide such a switch. There were only 2 switch positions, left tank or right tank , I assume.

To switch from left to right tank, use a mouse click on the left or right area of the switch.

- Fuel Cock

For the fuel cock I could not find any original documents. I have tried to make use of enlarged photographs to pick the details from different cockpits of that era to be able to provide such a switch. There were 3 switch positions, I assume. Lever left or right – i.e pointing towards the fuel piping – was the open position, lever pointing downwards – perpendicular to the piping – was the closed cock.

To switch from left to right open, use a mouse click on the left or right area of the switch. Center click closes the fuel cock.

Remark to panel designers: Fuel cock and tank selector switch also will work together only.
You can NOT use this gauges independently

- Fuel Status Gauge

The fuel status gauge by company Maximall is one of the early "Standard Gauges", which was built after some standardization. This gauge was produced with housings made of brass or steel with different scales for all kind of usage in large quantities. The pointer needle tip is covered with luminous paint. The main values are pronounced by black fields and the ciphers are shown in luminous paint too.

This fuel status gauge is calibrated in Liters, range is from 0 to 175 litters.

- Oil Pressure Gauge

The oil pressure gauge by company Maximall is one of the early "Standard Gauges", which was built after some standardization. This gauge was produced with housings made of brass or steel with different scales for all kind of usage in large quantities.

This oil pressure gauge is calibrated in Kilogramms per square Centimeter (old= kg pro qcm, new= kg/cm^2) , range is from 0 to 5 kg/cm^2.

- Coolant Temperature Gauge

I could not find pictures from a coolant temeprature gauge. Therefore I have tried to follow the description of an old standardization drawing and to detailed enlargements from different cockpit photographs in order to provide a coolant gauge in the style of this era.

This coolant temperature gauge is calibrated in degrees Celsius (°C) and ranges from 30 to 100 °C.

# Fuel Filter and Inspection Window for Fuel Flow

In the Ju F13 – as for almost any aircraft of the pioneer era – the fuel was pumped up to a drain tank. This tank was located in the upper part of the cockpit behind the pilots. From this tank the fuel was gravity feed through the piping and filter towards the carburetor and engine. The inspection window allowed the control of fuel flow.

No function provided in FS.

Trimmwheel

This is implemented for the FS only. The Ju F 13 did NOT have a trimm wheel, trimming instead was performed by pumping the fuel from main tanks into a trimm tank located in the tail of the fuselage.

Have fun!

_________________________________________________________________________________

Horst Weingärtner

horst.weingaertner@t-online.de