AIRCRAFT DATA & SPECIFICATIONS

C-404 Titan specifications

Type: 						Twin engine executive turboprop aircraft

Role: 						Search And Rescue & transport aircraft

Manufacturer: 					Cessna

Engine:						2 x Continental TSIO-520-E turboprop

Take-Off power: 					2 x 300 hp / 373 kW at 1900 RPM

Propeller: 					3 blade, constant speed full feathering reverse

Overall length: 					39 ft 1 in / 11.89 m

Overall height: 					13 ft 1 in / 4.01 m

Wing span: 					49 ft 6 in / 15.08 m

Wing area: 					23.48 sq.m

Wing loading: 					181 kg per sq.m

Power loading: 					4.19 kg per sq.cm

Weight:						9925 lbs / 4502 kg maximum weight

9850 lbs / 4468 kg maximum take off weight

9361 lbs / 4246 kg maximum landing weight

8501 lbs / 3856 kg maximum zero fuel weight

5033 lbs / 2283 kg standard empty weight

4892 lbs / 2219 kg maximum useful load

Fuel capacities: 					474 US gal / 3225 lbs / 1463 kg (maximum)

468 US gal / 3183 lbs / 1444 kg (usable)

Oil capacity: 					2.3 US gal / 15.6 lbs / 7.1 kg / 8.7 l per engine

Maximum cruise speed at sea level: 		246 KIAS / 456 km/h

Maximum operating speed (VMO): 		230 KIAS / 426 km/h

Minimum control speed (VMCA): 			90 KIAS / 167 km/h

Maneuvering speed (VA): 				163 KIAS / 302 km/h

Maximum flap ext speed (VFE): 			200 KIAS / 370 km/h

Maximum landing speed (VLE): 			180 KIAS / 333 km/h

Stall speed (gear & flaps up): 			105 KIAS / 194 km/h

Stall speed (gear & flaps dn): 			75 KIAS / 139 km/h

Rate of climb: 					2550 ft/min (when both engines are running)

1300 ft/min (when only one engine is running)

Service ceiling:					45000 ft (when both engines are running)

25200 ft (when only one engine is running)

Load factor: 					3.60 Gs / -1.44 Gs (when weight is 4216 kg)

3.31 Gs / -1.37 Gs (when weight is 4468 kg)

Maximum range: 					1153 nm / 2135 km (with standard fuel)

Endurance: 					7 hours and 5 minutes

Crew / pilots: 					2 pilots and up to 12 passengers

COCKPIT PANEL & CONSOLES

This package you’ve downloaded from the Internet, also contains a panel which

had been modified by photos and diagrams of the real one. Some of the FS9 virtual pilot’s actions

on this panel, effects on this C-404 Titan aircraft model in FS9, just like as real pilot’s

actions on the real aviation. The panel consists by three (3) screens. To activate / de-activate each one

of the following screens you must press Shift + 2 or Shift + 3. Additionally you can use one of the

following buttons on the panel to activate / de-activate each one of the screens.

Button A	 Button B 	Button C

The Main panel contains the main panel with most of the aircraft’s flight instruments,

The Throttle panel (FS default) which switch ON / OFF by pressing Shift + 2.

The Compass panel, which switch ON / OFF by pressing Shift + 3.

The Air Traffic Control screen seen by pressing the button A.

The Map view screen seen by pressing the button B.

The Kneeboard screen seen by pressing the button C.

PANEL INDEX

Please read very carefully the following index in order to understand in the very best way the panel’s

functions. After that, you can try them during the simulation.

01) Toggle buttons				26) Radio altimeter preset knob

02) Magnetic compass				27) Left engine’s torque indicator

03) Landing gear light indicator			28) Right engine’s torque indicator

04) Stall warning light indicator			29) Left engine’s prop RPM indicator

05) Speed brake light indicator			30) Right engine’s prop RPM indicator

06) OMI marker indicator lamp			31) Left engine’s inner turbine temperature

07) Analog Quartz clock				32) Right engine’s inner turbine temperature

08) Analog airspeed indicator			33) Left engine’s oil temperature / pressure

09) DME digital display indications			34) Right engine’s oil temperature / pressure

10) Analog altimeter indicator			35) Left fuel tanks percent indicator (total)

11) Flap position indicator				36) Right fuel tanks percent indicator (total)

12) Analog VOR 1 indicator			37) Left engine’s fuel flow indicator

13) Analog VOR 2 indicator			38) Right engine’s fuel flow indicator

14) Analog attitude indicator			39) Caution annunciator light indicator

15) Digital elevator trim indicator			40) Auto pilot control

16) Analog vertical speed indicator			41) COM1 digital display frequencies

17) Engine fuel shut switches 			42) NAV1 digital display frequencies

18) Engine ignition switches			43) NAV2 digital display frequencies

19) Analog turn indicator 				44) ADF digital display frequencies

20) Light switches				45) Transponder digital display frequencies

21) Analog radio magnetic indicator		46) Left & Right engine thrust levers

22) Elevator trim knob				47) Left engine propeller adjust lever

23) Heating switches				48) Right engine propeller adjust lever

24) Radio altimeter indicator			49) Left & Right engine fuel levers

25) Avionics / Master BAT switches		50) Reverse point for throttles

STARTING UP THE ENGINES

Starting the engines up is an quite easy procedure. First off all set your parking brakes ON. Now press

buttons Shift + 2 to see the Throttle panel which contains the engine’s throttle levers control, the props

pitch adjust and the fuel cutoff levers. By clicking with your mouse’s left button on the Left & Right

engine fuel levers (49), slide and set the levers forward to the most HIGH IDLE fuel position. If the

Left & Right engine fuel levers (49) are set at LOW IDLE fuel position, the engines will never start!

So, follow the engine startup procedure for your C-404 Titan turboprop powered,

by clicking your mouse’s left button on the Engine fuel shut switches (17) to DOWN position (the

figures #1 and #2 must be seen) just like the following screenshots are showing. These switches might

automatically set by sliding the Left & Right engine fuel levers (49), at HIGH IDLE fuel position.

Completing the previous procedure as described before, the turboprop

engine’s fuel valves should now be “opened” and ready for pre-heating.

Here comes the difficult part which will start the engines. Study the

following lines VERY carefully in order to understand the procedure

and do not have any problems with this situation. To turn on the left Nr 1

engine, the Nr 1 Engine ignition switch (18), should be turned left (counter

clockwise) by clicking on it, with the mouse’s left button, as it is shown at the

screenshot to set the switch at START position and keep your mouse left

button pressed until you’ll hear the engine rotating. An small orange colored

indicator light, signed as START VALV OPN, will be lighted during this

procedure. By keeping the mouse left button pressed on the ignition switch,

the engine’s start up action can be simulated. After starting up the left Nr 1

engine, release the mouse button to let the switch come back at the “neutral”

position. Replay the procedure for the right Nr 2 engine as well

TAXIING

At this moment the turboprop engines are “alive”, the throttle levers are set to IDLE position and the

gear brakes are ON. Set the flaps to FULL, reset the Analog altimeter indicator (10), neutralize the

Digital elevator trim indicator (15) by clicking on it. Check the Analog attitude indicator (14) to be sure

that it is encaged and Analog turn indicator (19) for malfunctions.

Slide the Left engine propeller adjust lever (47) and Right engine propeller adjust lever (48) to the most

increased position (forward). Release the parking gear brakes and apply thrust power by sliding the

Left & Right engine thrust levers (46) forward too gently and wait for the engines to reply. Do NOT

attempt violent throttle lever move, if you do not want to damage your C-404 Titan aircraft!

The pilot who taxies slowly always has control of the airplane and can stop whenever he chooses.

When taxiing, it is important that speed and use of brakes be held to a minimum and that all controls be

utilized to maintain directional control and balance. Taxiing over loose gravel or cinders should be

done at low engines speed to avoid abrasions and stone damage to the propeller tips. While taxing to

the line, check the flight controls . The best taxi speed is between 15 KIAS and 20 KIAS.

CAUTION

Do not exceed this speed limit unless of an emergency situation.

BEFORE TAKE OFF PREPARATIONS

Be sure that all the instruments are working within the limits. Line up and follow the proper checklist.

Set your gear brakes ON. Test the engines by applying MAX power on them. Slide full forward the

Left & Right engine thrust levers (46), let them 2-3 sec and retract them back to IDLE position to

ensure that the engines are rotating normally “by the numbers”. If engines are accelerating smoothly,

the airplane is ready for take off procedures.

TAKE OFF

C-404 Titan’s flaps position are the following: UP, 10and 40(or FULL) position.

Normal take-offs are performed with flaps 10. Flap settings 20or greater, are approved only for short

field take offs. Use of flap setting 40or more, is needed for minimum ground runs or for take off from

soft or rough fields. Use of flaps allows save use of slightly lower take off speeds than with flaps UP.

The lower speeds result in shortening the ground run and total distance over a 50 ft obstacle by

approximately 10%. However, this advantage will be lost if flaps UP speeds are used, or in high altitude

take offs in hot weather at maximum weight where climb would be marginal with 10flaps. Therefore,

the use of 10flaps is NOT recommended for takeoff over an obstacle at high altitudes in hot weather.

CAUTION

To complete a nice & easy take off, elevator MUST be well trimmed to avoid useless pitching

Elevator trimming is needed to perform a take off run “by the numbers”. If

you try to take off your aircraft without trimming, you might have difficulties

on setting up the C-404 Titan on the air. So, by clicking with your

mouse’s left button on the Elevator trimmer knob (22), set the elevator’s

trim on the desired level. You can check the elevator’s trim level on the

Digital elevator trim indicator (15). The best trim for take off is 2-4.

Set the flap lever as required. Apply wheel brakes and set the Left & Right engine thrust levers (46) to

MAX. Let the engines to reply and after 2-4 sec release the gear brakes. For a fully fuel loaded aircraft,

approaching about 105 KIAS (when using flaps 10) or 98 KIAS (when using flaps over 20) is the

right time to raise the nose SMOOTHLY to a take-off attitude. For an aircraft which is not fully

loaded with fuels, these take-off speeds will surely become lower.

Maintain this attitude and allow the aircraft to fly off the ground, which will normally occur between

135 KIAS and 150 KIAS. Set the aircraft’s nose up, as high as it is necessary to establish and maintain

a positive climb rate. At 200 ft AGL, retract the landing gear by pressing the G button on keyboard.

Raise wing flaps after gear has been retracted (watch out the Landing gear light indicator (3)) and the

aircraft is at safe altitude and airspeed OR after any obstacles are cleared off or when the aircraft is at

safe altitude and airspeed. The best angle to succeed that is when maintaining a take-off climb between

15 degrees and 25 degrees. When the aircraft’s airspeed reaches 160 KIAS, retard the throttles to 75%

RPM to 85% RPM (watch the Left engine’s torque indicator (27) gauge and the Right engine’s torque

indicator (28) gauge) and maintain a climb rate about 1500 ft/min until reaching recommended climb

schedule. By clicking with your mouse’s left button on the Elevator trimmer knob (22), reduce the

trim as required, to maintain a normal flight attitude.

CLIMBING

Normal climbs are performed with flaps at UP position and Left & Right engine thrust levers (46), at

full MAX position and at speeds depending 5 to 10 KIAS higher than best rate of climb speeds for the

best combination of performance and visibility. Power settings between 75% RPM and 85% RPM

and elevator trimming 1, will provide a comfortable climb rates at 110 KIAS to 120 KIAS for

intermediate altitude level-offs. Maximum thrust instrument climbs require extremely high pitch angles

and are not normally used for instrument departures. If condition require a maximum thrust climb,

maintain a climb indication until approaching the recommended climb airspeed and then adjust pitch to

maintain climb schedule. When a full loaded aircraft is used, the best recommended climb rate (2500

ft/min) is succeeded, if a 10 degree to 15 degree climb and 115 KIAS to 117 KIAS speed maintained,

setting as elevator trimming 4 checked on the Digital elevator trim indicator (15).

CRUISE

Normal cruising is performed between 65% and 75% of power. The engine RPM and corresponding

fuel consumption for various altitudes can be determined by using the official cruising charts. These cruising charts provide aircraft cruising performance, which includes time, distance and fuel required to fly as well as estimated cruise-climb altitude & service ceiling for a various of drag indices. To perform the best cruise behavior you should maintain your flying within the aircraft’s limitations.

BALANCING AND TRIMING

As you might feel when you’ll fly the C-404 Titan aircraft in FS9, this model can be

fully loaded with 540 US gallons of fuel. The left main tank contains 190 US gals, right main tank also

contains 190 US gals, 80 US gals at the left auxiliary tank and another 80 US gals at the right auxiliary

tank. This means that if you decide to load for example 170 US gals in the left main tank and only 80

US gals in the right main tank you’ll have balance problems that you should attempt to repair by trimming.

When the main flight line will be established apply as much left or right trim as it is needed to center

the aircraft’s balance. As the fuel getting lower you should trim again as much as it is needed.

By the same way pitch trimming should be applied each time you’ll try to climb or descend after

established in the desired altitude. Fully trim actions take place on the LANDING phase of flight as it is

shown and explained later.

You MUST trim your aircraft’s elevator, by clicking your mouse on the

Elevator trimmer knob (22), each time you select a new flight level. If you already fly with A/P aid you

don’t need that; the A/P does all the “dirty job” for you! But if you fly “by hand”...you will sooner or

later will have to exercise trimming! The trim “number” depends by many reasons: Altitude, airspeed,

crosswinds, temperature etc. For example: If a fully loaded C-404 is flying at 7500 ft, flaps UP,

gears UP, airspeed 230 KIAS the trim must set to 1, to remain in LEVEL flight.

Now, if the airspeed change from 230 KIAS to 200 KIAS, the trim must also change and set to 2.

If the airspeed change again to 140 KIAS, the trim must change to 4. Elevator trimming isn’t needed only for level flights.

For example: If descending with -500 ft/min to land with flaps 40, gears DWN, airspeed 100 KIAS to 105

KIAS, the Digital elevator trim indicator (15) should indicate 8 or even 9!

DESCENDING

After setting the NAV aids up to establish a heading to the desired airport, an official descend

chart should be followed. Maximum range descend charts determine fuel, time and distance required to

descend from altitude at IDLE thrust. For use is FS9 you do not need to follow up these charts but

you should make an effort to prevent over speed! Use the throttle as required to reduce the cruising

speed and perform an as normal as can be succeeded descend. The best speed to perform a normal

procedure descend is between 135 KIAS and 140 KIAS while VSI is established at -1000 ft/min.

Apply elevator trimming as required.

LANDING

Normal landing procedures should be used to land the C-404 Titan in FS9 environment

too. The before landing check is used when returning from a flight that takes the airplane away from

the home field. Complete the pre-landing checks before entering the traffic pattern, so that thereafter

you will be able to devote your undivided attention to traffic and landing. Don’t forget to set the

automatic pilot to OFF. All A/P switches must turned OFF to eliminate any possibility of accidental

engagement.

The traffic pattern and the rules for entering and flying it, are prescribed by local field regulations. At

the majority of the airfields, the traffic pattern is rectangular in shape. The pattern altitude may vary, but

generally is between 800 ft and 1000 ft AGL (Above Ground Level). For traffic and safe spacing

purposes, fly the pattern at 135 KIAS to 140 KIAS and 1250 RPM, with manifold pressures sufficient

to hold the desired airspeeds, but not in excess of 15 units.

While entering in the upwind leg of traffic pattern, you should operate the gear lever to the DOWN

position to activate the main & the front landing gear but DO NOT activate the gears above 195 KIAS

speed. If you raise gears above this speed, maybe will effect a serious damage and not have the

opportunity to fix it! Even the gear system (maybe) will be raised, you cannot be sure for the proper gear

locking no matter the three green lights on the instrument panel.

After you’ll be established on the main landing path circling towards the desirable runway, maintain a

descend glide path. Keep your airspeed between 125 KIAS and 135 KIAS while attempting to “enter”

at the landing procedure. At 125 KIAS, while you’re turning left to “enter” in the downwind leg, set

your flap lever to 10and kneed your airspeed reduced not above 125 KIAS. When flying into the base

leg your airspeed should be 115 KIAS and you should set flap lever at the 20. Finally, on your final

approach set flaps at 40position when your airspeed is about 100 KIAS to 110 KIAS. Trim 6

Proper base leg should be placed NOT MORE than 3 nm far from the desired rwy’s threshold

As soon as you establish the final line for landing you should attempt your landing. To make a

successful landing try to trim the aircraft in order to maintain a pitch angle by 5 degrees to 8 degrees.

Land the aircraft by sliding the Left & Right engine thrust levers (46), back to IDLE position or as back

as needed to make the aircraft “glide” and finally touch on the runway as smooth as possible. You can

increase / decrease the aircraft’s sink rate by setting the Left & Right engine thrust levers (46), at the

“right place” or by applying trim action on Elevator trimmer knob (22) as required and checking on the

Digital elevator trim indicator (15)! Personally, I prefer to control the aircraft’s sink rate by setting my

throttle levers on IDLE position and applying trim as required.

An easy way / trick to make a successful “final” approach and maintain in ideal glide path for landing is

to try to “aim” the desired runway’s threshold inside the upper part of your panel as it is shown at the

following main screenshot AND ALSO maintain a descent rate about -800 feet/min to -1000 feet/min.

Try to keep the Analog vertical speed indicator (16) needle, close to the “-800 ft/min” position as it is

shown at the following right screenshot. “Aim” the desirable runway’s threshold by using your

Elevator trimmer knob (22) (control stick or pitch trim) and the desirable glide rate by using the right

Left & Right engine thrust levers (46) setting.

When altitude will get lower than 100 ft to 125 ft AGL (watch on the Radio altimeter indicator (24)),

pull veeeery gently back the control stick, to establish a -500 ft/min maximum sink rate and you must

remain this until touchdown. Try to keep the Analog vertical speed indicator (13) needle, BELOW

the -800 ft/min limit as it is shown at the following left screenshot. The landing should be made with

as low a sink rate as possible. Maximum landing sink rate for a loaded aircraft is 800 ft/min for

normal landings and 600 ft/min for crosswind landings. The best landing speed is 90 KIAS to 95 KIAS

as shown at the following screenshots.

WARNING

You will NEVER – NEVER – NEVER make a safe landing if you try to touchdown when your

sink rate is more than -1000 ft/min and your airspeed is above 200 KIAS.

On all landings, take particular care to avoid holding brakes while using rudder on the approach.

Landing with brakes or applying brakes before the full weight of the airplane settles, will cause damage

to the landing gear system and the airplane might lose straight direction. After touchdown, hold the

nose as high as possible without becoming airborne, to obtain maximum aerodynamic braking. Hold

this attitude until speed is reduced to 50 KIAS. After nose wheel is lowered, apply brakes. When your

speed get lower than 20 KIAS to 25 KIAS release the continues brakes and apply brakes occasionally as

required. Apply as much braking strength is needed to decelerate and avoid locking on brakes. Tire

skidding on a wet runway will increase stopping distance and can easily result in loss of directional

control. Make every effort to remain in the center line of the runway if any barrier engagement should

become necessary. Decelerate to 15 KIAS and taxi to the airport’s apron.

GO AROUND

If airplane is not on the ground within the first 1/3 of the runway and the conditions to make a landing

or approach are unsafe, you must go around again and make another approach and landing. Make the

decision to go around as soon as possible. If touchdown is unavoidable, do not try to hold the aircraft

off the runway, but continue to fly the aircraft to touchdown. If a touchdown is make, lower the nose

slightly to a normal takeoff attitude and allow the aircraft to accelerate to takeoff. When a go-around is

required at low altitude, proceed as follows:

Throttle levers - MAX (both full forward position)

Flap lever - Retract to UP position after applying power. While they are being retracted, raise

nose slightly to overcome the loss of lift which occurs as flaps come fully up.

Maintain a minimum speed 100 KIAS and avoid using excessive bank angles at low altitudes because

stall speed increases as bank angle increase and sufficient altitude may not be available for recovery.

When you reach 1000 ft AGL, and if or as soon as conditions permit, set the flap lever to 10possition.

After that, you may now try a new landing pattern entry.

ENGINES SHUT DOWN

After taxiing back to apron (taxiing procedures explained before) stop the aircraft, set parking brakes

(in real aviation you shouldn’t apply parking brakes because the hot brakes may cause the expander

tubes to burst) and follow the “Engine shut down” procedure:

Retract the Left & Right engine thrust levers (46), back to IDLE position by setting the flight yoke’s or

joystick’s throttle lever “full back” or by pressing the F1 key on your keyboard and using the following

procedures, shut the engine down. Click with the mouse’s left button on the Left & Right engine fuel

levers (49) and drag them until setting the levers back to the most LOW IDLE position. As soon as

the Left & Right engine fuel levers (49) set at FUEL CUTOFF position, the engines will stop running!

Click your mouse’s left button on the Engine fuel shut switches (17) to UP position (the figures #1

and #2 must NOT be seen) just like the following screenshots are showing. Engine’s fuel valves are

now “closed” and normally both engines are not running. After both propels are finally stopped, be sure

that both left & right engine ignition switches are set at the “neutral” position as shown bellow:

ENGINES & SYSTEMS LIMITATION

Maximum speed limit (glide, dive or smooth air): 246 KIAS at sea level.

Flaps FULL maximum operating / damaging speed: 220 KIAS

Maneuvering speed *: 240 KIAS

( * ) The speed at which you can use abrupt control travel without exceeding the design load limit.

Stall speed : 105 KIAS when flaps UP, 75 KIAS when flaps FULL.

Controls: The aircraft cannot fly when above 45000 ft.

Nose wheel steering: Never turn the nose wheel more than 35° either side of center, or structural

nose gear damage could result.

Load factor limit: +3.2 G / -1.5 G when flaps UP and +2.5 G when flaps FULL.

Rudder: Never full abrupt rudder reverse deflection above 230 KIAS.

Nose wheel steering: The aircraft should not exceed 65 KIAS with nose wheel steering engaged.

Tires: The maximum tire speed limit is 200 KIAS ground speed. Do not attempt landings at this

speed!

Landing gear system: Retract wheels before 195 KIAS. Never exceed 220 KIAS if gears DOWN.