Piasecki H-21C & Vertol V-44 For Microsoft Flight Simulator X By Mick Posch 6-3-2007 Annotated Checklist Master Shutdown This helicopter is equipped with a master shutdown button, which will properly shut down all systems prior to a checklist startup. Just hit (Shift) 4 to bring up the button, and press it. It’s not a good idea to turn any switches on or off prior to using the master shutdown, because for certain functions it acts as a toggle. So, prior to doing a manual startup, the best procedure is to create a new flight, don't touch any switches, then shut down via the master shutdown. If you save the flight at that point, it will still be properly shut down when you reload it later. Switch Covers To operate switches that have red covers, click the cover to open it, click the switch to turn it on or off, then click the cover again to close it. Levers Most switches and levers are two-way toggles – click once to activate. A few, such as the fuel mixture lever and stick positioner, are click-and-drag. Camera View Keystrokes These need to be assigned; see the manual, under Assigning Camera Views. 4 – Engine Gauges 5 – Overhead Panel 6 – Center Console 7 – Pilot's View 8 – Floor - Right 9 – Floor - Left Useful Default Keystrokes Other useful keystrokes – already assigned by default – include: (Shift)2 – Radios (Shift)3 – GPS A – Toggle cameras within category S – Toggle between camera categories (cockpit , outside, and tower views) (Shift)E – Open/close front door (avail. only on civilian versions) (Shift)E then 2 – Open/close rear door (avail. only on civilian versions) F9 – Return to default eyepoint

This aircraft's real-world checklists have been modified for use with Flight Simulator

|  | Camera or Window | Key Stroke | Checklist | Remarks |
| --- | --- | --- | --- | --- |
| Preliminary | Master Shutdown | (Shift) 4 | Master shutdown button: ACTIVATE | Properly shuts down system prior to a checklist startup |

| Interior Check | - | - | Seat: ADJUST |  |
| --- | --- | --- | --- | --- |
|  |  |  | Safety belt & shoulder harness: FASTEN |  |
|  |  |  | Directional pedals: ADJUST (to comfortable position) |  |
|  |  |  | Flight Controls CHECK (for freedom of movement) |  |
|  | Overhead | 5 | Battery switch: ON |  |
|  |  |  | Dome light switch: ON (night flights) |  |
|  | Center Console | 6 | Collective Pitch Lever: DOWN & LOCKED | 1/4 turn to left |
|  | Floor - Left | 9 | Nosewheel UNLOCKED | Check - Lever in DOWN position |
|  | Floor - Right | 8 | Parking Brake: ON |  |

| Console Check | Center Console | 6 | Cyclic Trim: ZERO | Set Stick Positioner to zero |
| --- | --- | --- | --- | --- |
|  |  |  | Fuel mixture lever: IDLE CUTOFF |  |
|  |  |  | Carburetor heat lever: COLD |  |
|  |  |  | Oil temperature control switch: AUTO |  |
|  |  |  | Friction clutch switch: DISENGAGED (with warning light out) |  |
|  |  |  | Supercharger switch: LOW |  |
|  |  |  | Fuel Booster Pump Switch: OFF |  |
|  |  |  | Ignition switch: OFF |  |

| Overhead Panel Check | Overhead | 5 | Inverter switch: ON |  |
| --- | --- | --- | --- | --- |
|  |  |  | Generator switch: ON |  |
|  |  |  | Fire detection switch: TEST | -Press -Check main panel (4 key) - light should be on -Press again to turn off |
|  |  |  | Engine fluids switch: NORMAL |  |
|  |  |  | Pitot heat switch: OFF |  |
|  |  |  | Hoist switch: OFF |  |
|  |  |  | Heater switch: OFF |  |
|  |  |  | Blower switch: OFF |  |
|  |  |  | Navigation lights: OFF |  |
|  |  |  | Instrument lights: OFF |  |
|  |  |  | Windshield wipers: TEST |  |

| Starting Engine | Center Console | 6 | Fuel mixture lever: NORMAL |  |
| --- | --- | --- | --- | --- |
|  |  |  | Fuel booster pump switch: ON |  |
|  |  |  | Friction & jaw clutch switches: CHECK WARNING: Damage to the rotor system will result if starter is activated with friction or jaw clutch engaged | Both switches should be OFF |
|  | Start up | 6, then A | Starter switch: ENGAGE | The small orange switch on the underside of the collective |
|  | Center Console | 6 | Ignition switch: BOTH |  |
|  |  |  | Primer switch: AS NEEDED | Switch will stay in the ON position for a few moments, then return to OFF. Activate several times until engine is running smoothly. |
|  |  |  | Fuel mixture lever: RICH |  |
|  | Pressure & Temp. Gauges | 4 | Oil pressure & temperature: CHECK (stop engine if the oil pressure does not register within 10 seconds or reach 40 psi within 20 seconds | NOTE: Oil pressure and temperature need to be checked from the VC; these gauges are static in the 2-D cockpit |
|  | Overhead | 5 | Navigation lights: ON |  |

| Starting Rotors | Floor - Right | 8 | Parking Brake: ON |  |
| --- | --- | --- | --- | --- |
|  | Floor - Left | 9 | Nosewheel UNLOCKED | Check - Lever in DOWN position |
|  | Center Console | 6 | Collective Pitch Lever: DOWN & LOCKED |  |
|  |  |  | Directional pedals: Neutral position |  |
|  |  |  | Friction clutch: ENGAGE Lock switch in this position with the guard for all rotor operations | Friction clutch connects engine to rotor system |
|  | Pressure & Temp. Gauges | 4 | Rotor speed: VERIFY IN SYNC with engine speed | On the dual tachometer gauge, check that the rotor (small needle) and engine (large needle) have synchronized.* The numbers represent rpm x 100. *NOTE: Rotor engagement speed is not accurately simulated and will happen quickly |
|  | Center Console | 6 | Jaw clutch switch: ENGAGE CAUTION: Do not engage the jaw clutch switch before engine & rotor rpm have synchronized | Jaw clutch allows rotor to overrun engine (as required for autorotation) |

| Ground Tests | Floor - Left | 9 | Hydraulic pressure control valve: ON Check pressure 1000 psi |  |
| --- | --- | --- | --- | --- |
|  | Overhead | 5 | Generator output: CHECK (27 1/2 volts) | Volt meter is on upper right section of overhead panel - zoom out if necessary. |
|  | Center Console | 6 | Carburetor heat control lever: ADJUST as needed | Check the Carb Air Temperature gauge (4 key). If necessary, set lever to HOT to bring needle into green zone. |
|  | Pressure & Temp. Gauges | 4 | All temperature and pressure gauges: CHECK within green zone |  |
|  |  | (Shift) 2 | Radio: ON | This helicopter uses default Bendix King radios. Controls for the vintage radio equipment are also present, but are non-functional. |

| Taxiing Instructions | Floor - Right | 8 | Parking brake: OFF |  |
| --- | --- | --- | --- | --- |
|  | Pilot's View | F9 | Use the cyclic to move the helicopter forward or backward; use the pedals to provide directional control. Differential brakes should not be used to provide taxiing control except in emergency. To slow or stop the helicopter, displace the cyclic rearward, while increasing collective pitch. |  |

| Prior to Take-off | Floor - Left | 9 | Hydraulic pressure: CHECK 1000 psi |  |
| --- | --- | --- | --- | --- |
|  | Center Console | 6 | Fuel booster pump switch: ON |  |
|  |  |  | Supercharger switch: LOW |  |
|  |  |  | Fuel mixture lever: RICH |  |
|  |  |  | Cyclic Trim: CHECK | Stick Positioner set to ZERO |
|  |  |  | Crew & passengers: Alerted and ready for take-off |  |
|  |  |  | Area: CHECK (clear for take-off) |  |
|  | Floor - Right | 8 | Parking brake: OFF |  |

| Take-off | Center Console | 6 | Collective: UNLOCKED | 1/4 turn to right |
| --- | --- | --- | --- | --- |
|  | Pilot's View | F9 | Heading: Helicopter should be headed into the wind, when practical |  |
|  |  |  | Increase collective until airborne |  |
|  | Pressure & Temp. Gauges | 4 | All temperature and pressure gauges: CHECK within green zone |  |
|  |  | 6 | Carburetor air temperature: ADJUST if necessary |  |

| After Take-off |  |  | Cyclic trim: AS NEEDED | NOTE: The stick positioner is designed to visually replicate that of the real world H21, which is used for front-to-back trim. However, in the sim, its true function is to provide lateral trim (useful when dealing with FSX piston engine flight dynamics). The trim should be set to zero for hovering and slow flight. At cruising speed, it can be rotated forward to counteract FSX's left roll tendency - so that the stick doesn't need to be held to right of center to keep the roll attitude level. In tests at cruise speed, 1 1/2 was shown to be the best setting when flying at a general realism setting of 100%, 3/4 when at 50%, and it should be left at zero when at zero percent. |
| --- | --- | --- | --- | --- |
|  | Center Console | 6 | Fuel mixture lever: NORMAL |  |
|  |  |  | Carburetor heat control lever: ADJUST if necessary |  |

| Traffic Pattern, Approach, & Landing | Center Console | 6 | Landing light switch: ON (night flights) |  |
| --- | --- | --- | --- | --- |
|  | Pilot's View | F9 (or 7) | Landing area: CLEAR |  |
|  |  |  | Heading: Into the wind, when practical |  |
|  | Floor - Left | 9 | Nosewheel: UNLOCKED for taxiing | Check - Lever in DOWN position |

| Shutdown | Pressure & Temp. Gauges | 4 | Transmission temperature and pressure gauges: CHECK within green zone |  |
| --- | --- | --- | --- | --- |
|  | Center Console | 6 | Jaw clutch switch: DISENGAGE |  |
|  |  |  | Friction clutch switch: DISENGAGE | Rotors are now disengaged from the engine, and will take several minutes to stop turning. |
|  |  |  | Oil dilution: AS NEEDED CAUTION: Do not dilute oil when oil temperature is above 50 degrees C. | The oil dilution switch dilutes the engine oil with fuel, and is used when the next start will be made in cold temperatures. Maximum dilution time is 9 minutes; no other timing info is available. |
|  | Pressure & Temp. Gauges | 4 | Cylinder head temperature: CHECK Below 150 degrees C |  |
|  | Center Console | 6 | Fuel mixture lever: IDLE CUTOFF | Rotors will continue to turn after engine shutdown |
|  |  |  | Ignition switch: OFF |  |
|  |  |  | Center console - All switches: OFF |  |
|  | Floor - Left | 9 | Hydraulic pressure control valve: OFF |  |
|  | Floor - Right | 8 | Parking brake: ON |  |
|  | Center Console | 6 | Friction clutch switch: ENGAGE | At this point, the friction clutch switch is used as a rotor brake |
|  | Overhead | 5 | Overhead panel- All switches: OFF |  |